Packet내에 존재하는 Packet_count에 따라 10~15번째 Data는 다르게 분석하도록 한다.

| Packet_count | 0,4,8,….,252 | 1,5,9,….,253 | 2,6,10,….,254 | 3,7,11,…,255 |
| --- | --- | --- | --- | --- |
| 0 | Sync0(0xA5) |  |  |  |
| 1 | Sync1(0x5A) |  |  |  |
| 2 | 0x02 |  |  |  |
| 3 | Packet_count.(0 ~ 255) Increases by 1 each packet |  |  |  |
| 4 | Chn1high |  |  |  |
| 5 | Chn1low |  |  |  |
| 6 | Chn2high |  |  |  |
| 7 | Chn2low |  |  |  |
| 8 | Chn3high(PPG) |  |  |  |
| 9 | Chn3low(PPG) |  |  |  |
| 10 | Chn4high(Battery) | Acc_Xhigh | Gyro_Xhigh | 0 |
| 11 | Chn4low(Battery) | Acc_Xlow | Gyro_Xlow | 0 |
| 12 | 0 | Acc_Yhigh | Gyro_Yhigh | 0 |
| 13 | 0 | Acc_Ylow | Gyro_Ylow | 0 |
| 14 | 0x02 | Acc_Zhigh | Gyro_Zhigh | 0 |
| 15 | LeadOff Detail Info. | Acc_Zlow | Gyro_Zlow | 0 |
| 16 | Switches(LeadOff, 0x04:No Wear, 0x05: Wear) |  |  |  |

- Chnx : 12Bit Data

- Acc, Gyro : 16Bit Data

- LeadOff Detail Info : 각 Bit별 다음과 같은 의미를 갖는다.

[3] : LODN2, [2] : LODP2, [1] : LODN1, [0] : LODP1